View (M)SDS Section: 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16

SECTION 1: PRODUCT AND COMPANY IDENTIFICATION

Product Name:                                                                                           IMPA TIMBERCARE REACTION LACQUER

Product Code: 					VA6000, 6001, 6002

Company Name:					Lurama 149 (PTY) Ltd

Address:						P.O. Box 29270

Danhof

9301

Telephone:					(051) 432 8913

Facsimile:						(086) 661 3748

Contact Person:					Mr. J Strohfeldt

MSDS Creation Date:                                                                                 01 May 2014

SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

Chemical Class:					Wood Finishes

Classification:					F, Xn

R Phrases:						R11, 20/21, 33/38, 65

S Phrases:						S2, 16, 26, 36/7, 51

| Hazardous and/or other relevant Components | % (w/w) | CAS No | E.C. No |
| --- | --- | --- | --- |
| TOLUENE | >12,5% | 108-88-3 | 601-021-00-3 |
| XYLENE | >20% | 1330-20-7 | 601-022-00-9 |
| BUTANOL | <5% | 71-36-3 | 603-004-00-6 |

SECTION 3: HAZARDS IDENTIFICATION

SAPMA Health Rating:				2- MODERATE – Temporary or minor injury possible even if treatment given

Inhalation:					HARMFUL – Excessive exposure to this material must be avoided

Skin:						Highly irritating, Prolonged and repeated contact may cause dry skin and

dermatitis

Eye:						Irritating – may cause damage if not given prompt attention.

Ingestion:						Harmful – may cause lung damage if swallowed.

No carcinogenic, mutagenic or genetic effects established

May have short-term environmental effects; contain, monitor & remove.

SECTION 4: FIRST AID MEASURES

Eye contact:					Immediately flush eyes with plenty water for 15 to 20

minutes. Get medical attention, if irritation or

symptoms of overexposure persists

Skin contact:					Immediately wash skin with soap and plenty of water.

get medical attention if irritation develops or persists.

Inhalation:					If inhaled, remove to fresh air. If not breathing, give

Artificial respiration or give oxygen by trained personnel.

Seek immediate medical attention.

Ingestion:						If swallowed, do NOT induce vomiting. Call a physician or poison

control center immediately. Never give anything by mouth to an

unconscious person.

SECTION 5: FIRE FIGHTING MEASURES

Highly flammable with toxic fumes

Containers canburst in a fire.  Can form explosive vapour/air mixture.  Static discharge hazard.

In case of fire, use water fog, foam or dry powder.  Do NOT use water jets.

Cool containers in case of fire.

SECTION 6: ACCIDENTAL RELEASE MEASURES

Ensure sufficient ventilation

Contain & collect spillage with non-combustible absorbent material and dispose of according to local regulations

Do not allow product to enter drains and sewers.

| SECTION 7: HANDELING and STORAGE |  |  |
| --- | --- | --- |

Store separately from any reactive substances – oxidizers in particular

Keep away from ignition sources

Store in a well-ventilated, cool, dry environment away from direct sunlight or heat sources

Avoid free fall of liquid – use earthling

SECTION 8: EXPOSURE CONTOLS, PERSONAL PROTECTION – EXPOSURE GUIDELINES

| NAME | CAS | OEL (mg/m³) | OEL (ppm) | STOEL (mg/m³) | STOEL (ppm) |
| --- | --- | --- | --- | --- | --- |

(CL/RL = Controlled Recommended level ‘S’ = Skin annotation)

SAPMA Rating					2-H-E	PPE – Respirator and Eye protection

Inhalation:					In case of insufficient ventilation, use suitable respiratory

protection

Skin:						AVOID CONTACT.  Use barrier cream and impervious gloves

Eyes						Avoid any form of direct contact.  Use goggles is advised

Ingestion:						Observe the rules of hygiene.  Wash before eating, drinking or smoking

SECTION 9: PHYSICAL and CHEMICAL PROPERTIES

| Physical State Appearance: | Transparent Liquid |
| --- | --- |
| Flash Point: | <23◦C |
| Density: | -923 |
| Vapor Density: | Greater than 1 (Air = 1). |
| Viscosity: | 20 – 25 sec @23◦C (Fcup 4) |
| Molecular Formula: | Mixture |
| Molecular Weight: | Mixture |
| Solubility | Water miscible |
| SECTION 10: STABILITY and REACTIVITY |  |

Stable under normal conditions

Inert – no reaction with fire- fighting water

Avoid contact with acids, alkalis and hydrocarbon solvents

SECTION 11: TOXICOLOGICAL INFORMATION

No carcinogenic, mutagenic or genetic effects established

Toxicity levels not established

SECTION 12: ECOLOGICAL INFORMATION

Eco toxicity:					No eco toxicity data was found for the product.

Environmental Fate:	 				No environmental information found for this product.

SECTION 13: DISPOSAL CONSIDERATIONS

Use reputable waste disposal contractors. Exercise caution in disposal of used containers. Can be eliminated from sewage/ effluent by chemical flocculation.

SECTION 14: TRANSPORTATION INFORMATION

SIN/UN Number:					1263

Shipping Name: 					PAINT

Packaging Group:					II

Class:						3.2

SECTION 15: REGULATORY INFORMATION

Classification:					F, Xn

R11						Highly flammable

R20/21						Harmful by inhalation and in contact with skin

R36/38						Irritating to eyes and skin

R65						Harmful – may cause lung damage if swallowed

S2						Keep out of reach of children

S16						Keep away from sources of ignition – No smoking

S26						In case of contact with eyes, rinse immediately with plenty of water and seek

medical advise

S36/37						Wear suitable protective clothing and gloves

S51						Use only in well-ventilated areas

SECTION 16: ADDITIONAL INFORMATION

This Health and Safety Information is correct to the best of our knowledge and belief at the date of its publication but we cannot accept liability of any loss, injury or damage which result from its use. We shall ensure, so far as is reasonably practicable, that any revision of this Data Sheet is sent to all customers to whom we have directly supplied this substance, but must point out that It is the  responsibility of any intermediate supplier to ensure that such revision is passed to the ultimate user. The information given in the Data Sheet is designed only as guidance for safe handling, storage and the use of the substance. It is not a specification nor does it guarantee any specific properties. All chemicals should be handled only by competent personnel, without a controlled environment. Should further information be required, this can be obtained through the sales office whose address is at the top of this data sheet.

Trademark: The trademarks, service marks, graphics and logos used on this MSDS are registered or unregistered trademarks of IMPA PAINTS. All Rights Reserved