MATERIAL SAFETY DATASHEET

View (M)SDS Section: 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16

SECTION 1: PRODUCT AND COMPANY IDENTIFICATION

Product Name:                                                                                           RENOGLOSS ENAMEL

Product Code: 					RE2005

Company Name:					Lurama 149 (PTY) Ltd

Address:						P.O. Box

BFN

9300

Telephone:					(051) 432 8913

Facsimile:						(086) 661 3748

Contact Person:					Mr. J Strohfeldt

MSDS Creation Date:                                                                                 01 May 2014

SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

Chemical Class:					Alkyd based paint

Classification:					Flammable

R Phrases:						R10, 65

S Phrases:						S2, 13, 43, 62

| Hazardous and/r other relevant Components | % (w/w) | CAS no. | E.C No |
| --- | --- | --- | --- |
| HYDROCARBON BLEND (C9-C11 n-Paraffins | >10% | 64771-72-8 | - |

SECTION 3: HAZARDS IDENTIFICATION

SAPMA Health Rating:				3- SLIGHT – Irritation or minor reversible injury possible

Inhalation:					Irritating, narcotic, can affect CNS.

Skin:						Irritating, risk of dermatitis.  Avoid possible contact.

Eye:						Highly irritating.

Ingestion:						Harmful – may cause lung damage if swallowed.

No carcinogenic, mutagenic or genetic effects established

May have short-term environmental effects; contain, monitor & remove.

SECTION 4: FIRST AID MEASURES

Eye contact:					Immediately flush eyes with plenty water for 15 to 20

minutes. Get medical attention, if irritation or

symptoms of overexposure persists

Skin contact:					Immediately wash skin with soap and plenty of water.

get medical attention if irritation develops or persists.

Inhalation:					If inhaled, remove to fresh air. If not breathing, give

Artificial respiration or give oxygen by trained personnel.

Seek immediate medical attention.

Ingestion:						If swallowed, do NOT induce vomiting. Call a physician or poison

control center immediately. Never give anything by mouth to an

unconscious person.

MATERIAL SAFETY DATASHEET

SECTION 5: FIRE FIGHTING MEASURES

Flammable with toxic fumes.

Containers can burst in a fire.  Can form explosive vapour/air mixture.  Static discharge  hazard.

Use foam, CO2, dry powder.  Use fog to cool and control.  Do not use water jets.

Cool containers in case of fire.

SECTION 6: ACCIDENTAL RELEASE MEASURES

Ensure sufficient ventilation.

Contain & collect spillage with inert absorbent material and dispose of according to local regulations.

Keep out of drains and sewers.

| SECTION 7: HANDELING and STORAGE |  |  |
| --- | --- | --- |

Store separately from any reactive substances – oxidizers in particular

Keep away from ignition sources. No open flames. No smoking. Keep containers cool. Store away from direct sun and heat sources.

SECTION 8: EXPOSURE CONTOLS, PERSONAL PROTECTION – EXPOSURE GUIDELINES

| NAME | CAS | OEL (mg/m3) | OEL (ppm) | STOEL (mg/m3) | STOEL (ppm) |
| --- | --- | --- | --- | --- | --- |
| HYDROCARBON BLEND | 64771-72-8 | 350 | 200 | 2500 | - |

SAPMA Rating:					43-H-E PPE – Respirator and Eye protection

protective equipment.

Inhalation:					In case of insufficient ventilation, use suitable respiratory protection.

Skin:						Avoid direct contact.  Use barrier cream and impervious gloves.

Eyes:						Use goggles.  Avoid direct contact

Ingestion:						Observe the rules of hygiene. Wash hand before eating, drinking or smoking.

SECTION 9: PHYSICAL and CHEMICAL PROPERTIES

| Appearance: | White viscous liquid |
| --- | --- |
| Flash Point: | >23’C |
| Density: | 1.069 |
| Viscosity: | 68-82 KU |
| Solubility: | Soluble in organic solvents, insoluble in water |

SECTION 10: STABILITY and REACTIVITY

Stable under normal conditions

Inert – no reaction with fire- fighting water

Avoid contact with acids, alkalis and hydrocarbon solvents

SECTION 11: TOXICOLOGICAL INFORMATION

No carcinogenic, mutagenic or genetic effects established

MATERIAL SAFETY DATASHEET

SECTION 12: ECOLOGICAL INFORMATION

May have short-term environmental effects; contain, monitor & remove.

SECTION 13: DISPOSAL CONSIDERATIONS

Use reputable waste disposal contractors. Exercise caution in disposal of used containers. Can be eliminated from sewage/ effluent by chemical flocculation.

SECTION 14: TRANSPORTATION INFORMATION

SIN/UN Number:					1263

Shipping Name: 					Paint

Packaging Group:					III

Class:						3

SECTION 15: REGULATORY INFORMATION

Classification:

R10						Flammable

R65						Harmful – may cause lung damage if swallowed

S2						Keep out of reach of children. (Retail sales only)

S13						Keep away from food, drink and animal feeding stuffs

S43						Incase of fire, use water fog, foam or powder – Do not use water jets

S62						If swallowed, do not induce vomiting, seek medical advice immediately

SECTION 16: ADDITIONAL INFORMATION

Take precautionary measures against static discharges.  In case of any discomfort always seek medical advice. All information is given in good faith but without guarantee in respect of accuracy.  No responsibility is accepted for errors or omissions or the consequences thereof.