SECTION 1: COMPANY IDENTIFICATION

Company Name:					Prep it (PTY) Ltd

Address:						P.O. Box 11870

Universitas

9321

Telephone:						(051) 101 0769

Facsimile:						(086) 5345743

Contact Person:					Angelo Grobler

MSDS Creation Date:                                                                                 23 May 2014

SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

Hazardous Components Hydrochloric acid 28-31%

SECTION 3: HAZARDS IDENTIFICATION

Corrosive

Skin contact poisonous

Contact could cause burns to skin and eyes

Fire could produce irritating or poisonous gases

Reaction most metals with evolution of flammable hydrogen gas.

Runoff from fire –control or dilution water could cause pollution

SECTION 4: FIRST AID MEASURES

Eye contact:					Immediately flush eyes with plenty water for 15 to 20

minutes. Get medical attention, if irritation or

symptoms of overexposure persists

Skin contact:					Immediately wash skin with soap and plenty of water.

get medical attention if irritation develops or persists.

Inhalation:					If inhaled, remove to fresh air. If not breathing, give

artificial respiration or give oxygen by trained personnel.

Seek immediate medical attention.

Ingestion:						If swallowed, do NOT induce vomiting. Call a physician or poison

control center immediately. Never give anything by mouth to an

unconscious person.

SECTION 5: FIRE FIGHTING MEASURES

Flammable with toxic fumes.

Containers can burst in a fire.  Can form explosive vapour/air mixture.  Static discharge  hazard.

Use foam, CO2, dry powder.  Use fog to cool and control.  Do not use water jets.

Cool containers in case of fire.

SECTION 6: ACCIDENTAL RELEASE MEASURES

Ensure sufficient ventilation.

Contain & collect spillage with inert absorbent material and dispose of according to local regulations.

Keep out of drains and sewers.

| SECTION 7: HANDELING and STORAGE |  |  |
| --- | --- | --- |

Store separately from any reactive substances – oxidizers in particular

Keep away from ignition sources. No open flames. No smoking. Keep containers cool. Store away from direct sun and heat sources.

SECTION 8: EXPOSURE CONTOLS, PERSONAL PROTECTION – EXPOSURE GUIDELINES

| NAME | CAS | OEL (mg/m³) | OEL (ppm) | STOEL (mg/m³) | STOEL (ppm) |
| --- | --- | --- | --- | --- | --- |
| 2- BUTOXY ETHANOL | 111-76-2 | 120 | 25 CL S | - | - |

SAPMA Health Rating:				4-H-C PPE – Eye protection

Inhalation:					In case of insufficient ventilation, use suitable respiratory protection

Skin:						Avoid contact as far as possible.  Wash immediately after contact.  Use of 							gloves recommended

Eye:						Use goggles. Avoid direct contact.

Ingestion:						Observe the rules of hygiene.  Wash before eating, drinking or smoking.

SECTION 9: PHYSICAL and CHEMICAL PROPERTIES

| Appearance: | White viscous liquid |
| --- | --- |
| Flash Point: | >23’C |
| Density: | 1.030 |
| Viscosity: | 68-72 KU |
| Solubility: | Soluble in organic solvents, insoluble in water |

SECTION 10: STABILITY and REACTIVITY

Stable under normal conditions

Inert – no reaction with fire- fighting water

Avoid contact with acids, alkalis and hydrocarbon solvents

SECTION 11: TOXICOLOGICAL INFORMATION

No carcinogenic, mutagenic or genetic  effects established

SECTION 12: ECOLOGICAL INFORMATION

May have short-term environmental effects; contain, monitor & remove.

SECTION 13: DISPOSAL CONSIDERATIONS

Use reputable waste disposal contractors. Exercise caution in disposal of used containers. Can be eliminated from sewage/ effluent by chemical flocculation.

SECTION 14: TRANSPORTATION INFORMATION

Un No 1789 Hazchem Code 2r

ERG No 157 EAC 60

ARD/RID Class 8 ARD/RID Hazard ID No 5 (b)

IMDG – Shipping Name 8

IMDG Code 8174 IMDG-Packaging Group III

Marine Pollutant True

Class Class:  8 Corrosive Group: III

Subsidiary Risks None

Tremcard Number 50A/50B

SECTION 15: REGULATORY INFORMATION

Classification:					EEC Hazard Classification 8 Corrosive

R34						Causes burns

R37						Irritating to respiratory system

S2						Keep out of reach of children.

S26						In case of contact with eyes, rinse immediately with plenty of water and seek 						medical advice

SECTION 16: ADDITIONAL INFORMATION

Take precautionary measures against static discharges.  In case of any discomfort always seek medical advice. All information is given in good faith but without guarantee in respect of accuracy.  No responsibility is accepted for errors or omissions or the consequences thereof.