SECTION 1: COMPANY IDENTIFICATION

Company Name:					Prep it (PTY) Ltd

Address:						P.O. Box 11870

Universitas

9321

Telephone:						(051) 101 0769

Facsimile:						(086) 5345743

Contact Person:					Angelo Grobler

MSDS Creation Date:                                                                                  23 May 2014

SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

Chemical Class:					Mineral powders mixture

Hazardous components:				Contains quartz, do not inhale dust

Classification:					Non hazardous

R Phrases:						None

S Phrases:						S22

SECTION 3: HAZARDS IDENTIFICATION

SAPMA Health Rating:				3-SLIGHT – Irritation or minor reversible injury possible

Inhalation:					Irritating, avoid excessive exposure to dust

Skin:						Irritating, risk of dermatitis

Eyes:						Irritating

Ingestion:						Harmful.

No carcinogenic, mutagenic or genetic effects established

SECTION 4: FIRST AID MEASURES

Inhalation:					Move to fresh air. In case of discomfort seek medical attention.

Skin:						Use a skin cleaner/soap and water.  Avoid the use of solvent.

Eyes:						Rinse immediately with plenty of water.  Seek medical advice if symptoms

persist.

Ingestion:						Wash mouth with plenty of water.  Do not induce vomiting. Seek

immediate medical attention.

SECTION 5: FIRE FIGHTING MEASURES

Non- combustible

SECTION 6: ACCIDENTAL RELEASE MEASURES – SEE ALSO SECTIONS 5, 8, 13

Avoid inhalation of dust.  Contain & collect spillage with inert absorbent material and dispose of according to local regulations.  Keep out of drains, sewers and watercourses.

SECTION 7: HANDLING OF STORAGE

Avoid creating dust.  Avoid contact with eyes and inhalation of dust.  Keep in original package.   Keep in a closed container.  Avoid exposure to moisture before use.

SECTION 8: EXPOSURE CONTROLS/PERSONAL

| CONTROL LIMIT | Total weighted Average |
| --- | --- |
| QUARTZ | 0,4 mg.m-3 |

SAPMA Rating:					3-H-E Slight.  PPE – Respirator and Eye protection

Inhalation:					In case of insufficient ventilation, use suitable respiratory proection.

Skin:						Avoid direct contact.  Use barrier cream and impervious gloves.

Eyes:						Use goggles.  Avoid direct contact.

Ingestion:						Observe the rules of hygiene.  Wash before eating, drinking or smoking.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

| Appearance: Flash Point: | Off white powder Non flammable |
| --- | --- |
| Density: | 2.56 (typical) |
| Solubility: | Water miscible |

SECTION 10: STABILITY AND REACTIVITY

Keep away from moisture that cause the mixture to harden

SECTION 11: TOXICOLOGICAL INFORMATION

No carcinogenic, mutagenic or genetic effects established

SECTION 12: ECOLOGICAL INFORMATION

May have short-term environmental effects; contain, collect & remove.  							             Keep away from sewers and watercourses.

SECTION 13: DISPOSAL INFORMATION

Dispose of according to local regulations.

SECTION 14: TRANSPORT INFORMATION

SIN/UN Number:					Not classified

Shipping Name: 					Not classified

Packaging Group:					Not applicable

Class:						Not applicable

SECTION 15: REGULATORY INFORMATION

Classification:					Non hazardous

S22						Do not breath dust

SECTION 16: OTHER INFORMATION

In case of any discomfort always seek medical advice.

All information is given in good faith but without guarantee in respect of accuracy.  No responsibly is accepted for errors or omissions or the consequences thereof.