SECTION 1: COMPANY IDENTIFICATION

Company Name:					Prep it (PTY) Ltd

Address:						P.O. Box 11870

Universitas

9321

Telephone:						(051) 101 0769

Facsimile:						(086) 5345743

Contact Person:					Angelo Grobler

MSDS Creation Date:                                                                                 23 May 2014

SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

Chemical Class:					Water based polymer dispersion

Classification:					Not classified as hazardous

R Phrases:						None

S Phrases:						S2, 13

| Hazardous and/or other relevant Components | % (w/w) | CAS No. | E.C. No. |
| --- | --- | --- | --- |
| 2- BUTOXY ETHANOL | <12.5 | 111-76-2 | 603-014-00-0 |

SECTION 3: HAZARDS IDENTIFICATION

SAPMA Health Rating:				4-MINIMAL – No great risk to health

Inhalation:					May cause irritation

Skin:						No hazard expected.  Direct contact best to be avoided

Eyes:						May cause irritation.  Avoid any possible contact

Ingestion:						Irritating and nauseating.

No carcinogenic, mutagenic or genetic effects established.

May have short-term environmental effects; contain, monitor & remove.

SECTION 4: FIRST AID MEASURES

Inhalation:					Move to fresh air.   In case of discomfort seek medical attention.

Skin:						Wash with soap and water or a recognized skin cleaner

Eyes:						Rinse immediately with plenty of water.  Seek medical advice if symptoms

persist.

Ingestion:						Rinse mouth with plenty of water.  Do not induce vomiting. Seek

medical attention.

SECTION 5: FIRE FIGHTING MEASURES

The product is non-flammable.  no direct explosion hazard expected

In case of fire, use foam, CO2 or dry powder

SECTION 6: ACCIDENTAL RELEASE MEASURES – SEE ALSO SECTIONS 5, 8, 13

Contain & collect spillage with inert, absorbent material and dispose of according to local regulations.

Do not allow product to enter drains and sewers.

SECTION 7: HANDLING OF STORAGE

Store in a cool environment away from direct sun and heat sources.

Keep away from strongly alkaline materials and reactive substances.

SECTION 8: EXPOSURE CONTROLS/PERSONAL

| NAME | CAS | OEL (mg/m3) | OEL (ppm) | STOEL (mg/m3) | STOEL (ppm) |
| --- | --- | --- | --- | --- | --- |
| 2- BUTOXY ETHANOL | 111-76-2 | 120 | 25 CL S | - | - |

SAPMA Rating:					4-H-C  PPE – EYE protection

Inhalation:					In case of insufficient ventilation, use suitable respiratory protection.

Skin:						Avoid contact as far as possible.  Wash immediately after contact.  Use of

gloves recommended.

Eyes:						Avoid direct contact.  Never touch eyes with dirty hands or gloves.

Ingestion:						Observe the rules of hygiene.  Wash before eating, drinking or smoking.

| SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |
| --- | --- | --- | --- |
| Appearance: FLASH Point: | Milky white opaque liquid N/A |
| Density: | 1.08 (typical) |
| Viscosity: | 60 – 65 KU |
| pH: Solubility: | 1.5 – 3.2 Water miscible |
| SECTION 10: STABILITY AND REACTIVITY |  |  |  |

Stable under normal conditions.  Inert – no reaction with fire-fighting water.

Avoid contact with strong alkalis

SECTION 11: TOXICOLOGICAL INFORMATION

No carcinogenic, mutagenic or genetic effects established.

SECTION 12: ECOLOGICAL INFORMATION

May have short-term environmental effects.  Contain monitor and remove.

SECTION 13: DISPOSAL INFORMATION

Use reputable waste disposal contractors and dispose off accordance with local regulations.

Can be eliminated from sewerage/effluent by chemical flocculation.

SECTION 14: TRANSPORT INFORMATION

SIN/UN Number:					Not classified as hazardous for transport

Shipping Name: 					Not classified

Packaging Group:					Not classified

Class:						Not classified

SECTION 15: REGULATORY INFORMATION

Classification:					Not classified as hazardous

S2						Keep out of reach of children

S13						Keep away from food, drink and animal feeding stuffs

SECTION 16: OTHER INFORMATION

In case of any discomfort always seek medical advice.

All information is given in good faith but without guarantee in respect of accuracy.  No responsibly is accepted for errors or omissions or the consequences thereof.