SECTION 1: COMPANY IDENTIFICATION

Company Name:					Prep it (PTY) Ltd

Address:						P.O. Box 11870

Universitas

9321

Telephone:						(051) 101 0769

Facsimile:						(086) 5345743

Contact Person:					Angelo Grobler

MSDS Creation Date:                                                                                 23 May 2014

SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

Substance/mixture:					Mixture

Other means of identification:				None

CAS number/other identifiers

CAS number:					Not applicable

EC number:					Mixture

Product code:					None assigned

| Ingredient name | % | CAS no. |
| --- | --- | --- |
| Methylene chloride | 60 - 80 | 75-09-2 |
| Ethanol | 20 – 30 | 64-17-5 |
| mix-cresol | <5 | 1319-77-3 |

There are no additional ingredients present which, within the current knowledge of the supplier and in the concentrations applicable, are classified as hazardous to health or the environment and hence require reporting in this section.

Occupational exposure limits, if available, are listed in Section 8

SECTION 3: HAZARDS IDENTIFICATION

Respone:						IF exposed or concerned:  Get medical attention.  IF SWALLOWED:  Call a

POISON CENTER or physician if you feel unwell.  Rinse mouth.  IF ON SKIN:  Wash with plenty of soap and water   Take off contaminated clothing.  Wash contaminated clothing before reuse.  If skin irritation occurs:  Get medical attention.  IF IN EYES:  Rinse cautiously with water for several minutes.  Remove contact lenses, if present and easy to do.  Continue rinsing.  Immediately call a POISON CENTER or physician.

Storage:						Store locked up.

Disposal:						Dispose of contents and container in accordance with all local, regional,

national and international regulations.

Other hazards which do not result in classification:		None identified

SECTION 4: FIRST AID MEASURES

Eye contact:					Get medical attention immediately.  Call a poison center or

Skin contact:					Immediately wash skin with soap and plenty of water, occasionally lifting

The upper and lower eyelids.  Check for and remove any contact lenses.

Continue to rinse for at least 10 minutes.  Chemical burns must be treated promptly by a physician.

Inhalation:					Get medical attention immediately.  Call a poison center or physician.

Remove victim to fresh air and keep at rest in a position comfortable for

breathing.  If it is suspected that fumes are still present rescuer should wear

an appropriate mask or self-contained breathing apparatus.  If not breathing,

if breathing is irregular or if respirator arrest occurs, provide artificial

respiration or oxygen by trained personnel.  It may be dangerous to the

person providing aid to give mouth-to-mouth resuscitation.  If unconscious,

place in recovery position and get medical attention immediately.  Maintain

and open air way.  Loosen tight clothing such as a collar, tie, belt or

waistband.  In case of inhalation of decomposition product in a fire,

symptoms may be delayed.  The exposed person may need to be kept under

medical surveillance for 48 hours

Skin contact:					Get medical attention immediately.  Call a poison center or physician.  Flush

contaminated skin with plenty of water.  Removed contaminated clothing

and shoes.  Wash contaminated clothing thoroughly with water before

removing it, or wear gloves.  Continue to rinse for at least 10 minutes.

Chemical burns must be treated promptly by a physician.  Wash clothing

before reuse.  Clean shoes thoroughly before reuse.

Ingestion:						Get medical attention immediately.  Call a poison center or physician.

Wash out mouth with water.  Remove dentures if any.  Remove victim to

fresh air and keep at rest in position comfortable for breathing.  If material

has been swallowed and the exposed person is conscious, give small

quantities of water to drink.  Stop if the exposed person feels sick as vomiting

may be dangerous.  Do not induce vomiting unless directed to do so by

medical personnel.  If vomiting occurs, the head should be kept low so that

vomit does not enter the lungs.  Chemical burns must be treated promptly by

a physician.  Never give anything by mouth to an unconscious person.   If

unconscious, place in recovery position and get medical immediately.

Maintain and open airway.  Loosen tight clothing such as a collar, tie, belt

or waistband.

Most important symptoms/effects, acute and delayed

Potential acute health effects

Eye contact:					Causes serious eye damage

Inhalation:					May give off gas, vapor or dust that is very irritating or corrosive to the

respiratory system.  Exposure to decomposition products may cause a health

hazard.  Serious effects may be delayed following exposure

Skin contact:					Causes skin irritation

Ingestion:						Harmful if swallowed.  May cause burns to mouth, throat and stomach.

Over-exposure signs/symptoms

Eye contact:					Adverse symptoms may include the following:  pain, watering, redness

Inhalation:					No specific data

Skin contact:					Adverse symptoms may include the following:  pain or irritation, redness

blistering may occur

Ingestion:						Adverse symptoms may include the following:  stomach pains

Indication of immediate medical attention and secial treatment needed if necessary

Notes to physician:					In case of inhalation of decomposition products in a fire, symptoms may be

delayed.  The exposed person may need to be kept under medical

surveillance for 48 hours.

Specific treatments:					No specific treatments

Protection of first-aiders:				No action shall be taken involving any personal risk or without suitable

training.  If it is suspected that fumes are still present, the rescuer should

wear an appropriate mask or self-contaminated breathing apparatus.

It may be dangerous t the person providing aid to give mouth-to-mouth

resuscitation.  Wash contaminated clothing thoroughly with water before removing it, or wear gloves.

SECTION 5: FIRE FIGHTING MEASURES

Flammable with toxic fumes.

Containers can burst in a fire.  Can form explosive vapour/air mixture.  Static discharge  hazard.

SECTION 6: ACCIDENTAL RELEASE MEASURES

Personal precautions, protective equipment and emergency procedures

For non-emergency personnel:				No action shall be taken involving any personal risk or without suitable

training.  Evacuate surrounding areas.  Keep unnecessary and unprotected

personnel from entering.  Do not touch or walk through spilled material.  Do

not breathe vapor or mist.  Provide adequate ventilation

For emergency responders:				If specialized clothing is required to deal with the spillage, take note of any

information in Section 8 on suitable and unsuitable materials.  See also the information in “For non-emergency personnel”

Environmental precautions:;				Avoid dispersal of spilled materials and runoff and contact with soil,

waterways, drains and sewers.  Inform thee relevant authorities if the

product has caused environmental pollution (sewers, waterways, soil or air).

Water polluting material.  May be harmful to the environment if released in

large quantities.

Methods and materials for containment and cleaning up

Small spill:						Stop leak if without risk.  Move containers from spill area.  Dilute with water

and mop up if water-soluble.  Alternately, or if water-soluble, absorb with an

inert dry material and place in an appropriate waste disposal container.

Dispose of via a licensed waste disposed contractor.

Large spill:						Stop leak if without risk.  Move containers from spill area. Approach release

from upwind.  Prevent entry into sewers, water courses, basements or

confined areas.  Wash spillages into an effluent treatment plant or proceed

as follows.  Contain and collect spillage with non-combustible, absorbent

material e.g. sand, earth, vermiculite or diatomaceous earth and place in

container for disposal according to local regulations (see section 13).  Dispose

of via a licensed waste disposal contractor.  Contaminated absorbent

material may pose the same hazard as the spilled product.  Note:  see section 						1 for emergency contact information and section 13 for waste disposal.

| SECTION 7: HANDELING and STORAGE |  |  |
| --- | --- | --- |

Precautions for safe handling:				Put on appropriate personal protective equipment (see section 8).  Eating,

drinking and smoking should be prohibited in areas where this material is

handled, stored and processed.  Workers should wash hands and face before

eating, drinking and smoking.  Remove contaminated clothing and protective

equipment before entering eating areas.  Avoid exposure – obtain special

instructions before use.  Do not handle until all safety precautions have been read and understood.  Do not get in eyes or on skin or clothing.  Do not breathe vapor or mist.  Do not ingest.  Avoid release to the environment.  If during normal use the material presents a respiratory hazard use, only with adequate ventilation or wear appropriate respirator. Keep in the original container or an approved alternative made from a compatible material, kept tightly close when not in use.  Empty containers retain product residue and can be hazardous.  Do not reuse container

Conditions for safe storage, including any

Incompatibilities:					Store in accordance with local regulations.  Store in original container

protected from direct sunlight in a dry, cool and well-ventilated area, away

from incompatible materials ( see section 10) and food and  drink.  Store

locked up.  Keep container tightly closed an sealed until ready for use.

Containers that have been opened must be carefully resealed and kept

upright to prevent leakage.  Do not store in unlabeled containers.  Use

appropriate containment to avoid environmental contamination.

SECTION 8: EXPOSURE CONTOLS, PERSONAL PROTECTION – EXPOSURE GUIDELINES

Control parameters

Occupational exposure limits

Methylene						ACGIH (United States, 1996)

TWA: 50 ppm

TWA: 200 mg/m³

CEIL: 250 mg/m³

ACGIH TLV (United States, 2/2010)

TWA: 50 ppm 8 hour(s)

TWA: 174 mg/m³ 8 hour(s)

ACGIH (United States, 1/2014). Notes: Substance identified by other

sources as a suspected or confirmed human carcinogen.  1996 Adoption

Substances for which the TLV is higher than the OSHA Permissible Exposure

Limit (PEL) and/or the NIOSH Recommended Exposure Limit (REL).  See CFR

58 (124):36338-33351, June 30, 1993, for revised OSHA PEL.  Refers to

Appendix A – Carcinogens.

TWA: 174 mg/m³ 8 hour (s).  Form:  All forms

TWA: 50 ppm 8 hour (s).  Form:  All forms

Ethanol:						ACGIH (United States, 1996)

TWA: 1000 ppm

ACGIH TLV (United States, 2/2010).  Notes:  1996 Adoption Refers to

Appendix A Carcinogens.

STEL: 1000 ppm 15 minute(s)

Mix-cresol:					ACGIH TLV (United States, 2/2010)

Absorbed through skin.

TWA : 20 mg/m³ 8 HOUR (S).  Form:  Inhalable fraction and vapor

ACGIH (United States, 1994).  Absorbed through skin.

TWA:  5 ppm

TWA: 22 mg/m³

Recommended monitoring

Procedures:					If this product contains ingredients with exposure limits, personal, workplace

atmosphere or biological monitoring may be required to determine the

effectiveness of the ventilation or other control measures and/or the

necessity to use respiratory protective equipment.

Appropriate engineering controls:			If user operations generate dust, fumes, gas, vapor or mist, use process

enclosures, local exhaust ventilation or other engineering controls to keep

worker exposure to airborne contaminants below any recommended or

statutory limits.

SECTION 9: PHYSICAL and CHEMICAL PROPERTIES

| Appearance: | Pale liquid |
| --- | --- |
| Flash Point: | >23’C |
| Density: | 1.14 |
| Solubility: | Soluble in organic solvents, insoluble in water |

SECTION 10: STABILITY and REACTIVITY

Stable under normal conditions

Inert – no reaction with fire- fighting water

Avoid contact with acids, alkalis and hydrocarbon solvents

SECTION 11: TOXICOLOGICAL INFORMATION

Acute toxicity

Product/ingredient name			Result			Species		Dose		Exposure

methylene chloride				LC 50 Inhalation Gas		Mouse		14400 ppm	7 hours

LC50 Inhalation Vapor		Rat		76000 mg.m³	4 hours

LC50 Intraperitoneal		Rat		916 mg/kg		-

LD50 Oral			Rat		985 mg/kg		-

LD50 Oral			Rat		1600 mg/kg	-

LDlo Oral			Human – Male	357 mg/kg		-

Ethanol					LC50 Inhalation Vapor		Rat		124700 mg/m³	4 hours

LD50 Oral			Rat		7 g/kg		-

mix-cresol					LD50 Dermal		Rabbit		200 mg/kg		-

LD Oral			Rat		1454 mg/kg	-

Irritation Corrosion

Product/ingredient name		Result			Species	Score	Exposure		Observation

methylene chloride			Eyes-Mild irritant		Rabbit	-	24 hours 500 milligrams	-

Eyes-Mild irritant		Rabbit	-	10 milligrams 		-

Eyes- Moderate irritant	Rabbit	-	162 milligrams		-

Skin-Moderate irritant	Rabbit	-	24 hours 100 milligrams	-

Skin-Severe irritant		Rabbit	-	24 hours 810 milligrams	-

Ethanol				Eyes-Mild irritant		Rabbit	-	24 hours 500 milligrams	-

Eyes-Moderate irritant	Rabbit	-	0.066666667 minutes 100

Milligrams			-

Eyes-Moderate irritant	Rabbit	-	100microliters		-

Eyes-Moderate irritant	Rabbit	-	100microliters		-

Eyes-Moderate irritant	Rabbit	-	100microliters		-

Eyes-Severe irritant		Rabbit	-	500milligrams		-

Skin-Mild irritant		Rabbit	-	400 milligrams		-

Skin-Moderate irritant	Rabbit	-	24 hours 20 milligrams	-

Sensitization

No specific data

Mutagenicity

No specific data

Carcinogenicity

No specific data

Reproductive toxicity

No specific data

Teratogenicity

No specific data.

Specific target organ toxicity (single exposure)

No specific data.

Aspiration hazard

No specific data.

Information on the likely routes of exposure:			Not available

Potential acute health effects

Eye contact:						Causes serious eye damage.

Inhalation 							May give off gas, vapor or dust that is very irritating or corrosive

to the respiratory  system.  Exposure to decomposition products

may cause a health hazard.  Serious effects may be delayed

following exposure.

Skin contact:						Causes skin irritation

Ingestion:							Harmful if swallowed.  May cause burns to mouth, throat and

Stomach

Symptoms related to the physical, chemical and toxicological characteristics

Eye contact:						Adverse symptoms may include the following: pain, watering,

Redness

Inhalation:						No specific data

Skin contact:						Adverse symptoms may include the following: pain, or irritation,

redness, blistering may occur

Ingestion:							Adverse symptoms may include the following: stomach pains

Delayed and immediate effects and also effects from short and long term exposure

Short term exposure

Potential immediate effects:						No specific data

Potential delayed effects:						No specific data

Long term exposure

Potential immediate effects:						No specific data

Potential delayed effects:						No specific data

Potential chronic health effects

General:								No known significant effects or critical hazards

Carcinogenicity:							Suspected of causing cancer.  Risk of cancer depends

on duration and level of exposure.  Methylene chloride

classified category 2b (possibly carcinogenic to

humans) by IARC

Mutagenicity:							No known significant effects or critical hazards.

Teratogenicity:							No known significant effects or critical hazards

Developmental effects:						No known significant effects or critical hazards

Fertility effects:							No known significant effects or critical hazards

Numerical measures of toxicity

Acute toxicity estimates

Route							ATE value

Oral							970 mg/kg

Dermal							6666.7 mg/kg

SECTION 12: ECOLOGICAL INFORMATION

Toxicity

| Product/ingredient name | Result | Species | Exposure |
| --- | --- | --- | --- |
| Methylene chloride Ethanol Mix-cresol | Acute EC 50 242 mg/L fresh water Acute EC >500000 ug/L Fresh water Acute EC 50 99000 ug/L Fresh water Acute LC 50 108500 ug/L Marine water Acute LC 50 220000 ug/L Fresh water Acute EC50 17.921 mg/L Marine water Acute EC50 2000 ug/L Fresh Water Acute LC 50 25500 ug/L Marine water Acute LC 50 4200 ug/L Fresh water Chronic NOEC 0.375 ul/L Fresh water Acute EC 50 5 to 10 ppm Marine water Acute EC 50 7000 ug/L Fresh water Acute LC 50 10000 ug/L Fresh water | Algae – Chlamydomonas Reinhardtii – Exponential growth phase – 7 days Algae – Pseudokirchneriella subcapitata Fish – Pimephales promelas – 49mm – 1.04g Crustaceans – Palaemonetes pugio – Juvenile (Fledgling, Hatchling, Weanling) – 20 mm Daphnia – Daphnia magna - <= 24 hours Algae – Ulva pertusa Daphnia – Daphnia magna Crustaceans – Artemia franschiscana – Larvae Fish – Oncorhynchus mykiss Fish – Gambusia holbrooki – Larvae – 3 days Algae – Macrocystis pyrifera – Young Crustaceans – Gammarus fasciatus Fish – Lepomis macrochirus – 14.24 cm – 54.26g | 72 hours 96 hours 96hours 48 hours 48 hours 96 hours 48 hours 48 hours 4 days 12 weeks 4 days 48 hours 96 hours |

Persistence and degradability

| Product/ingredient name | Test | Result | Dose | Inoculum |
| --- | --- | --- | --- | --- |
| Ethanol: ethyl alcohol | - | 55% - Inherent – 5 days | - | - |

SECTION 13: DISPOSAL CONSIDERATIONS

Use reputable waste disposal contractors. Exercise caution in disposal of used containers. Can be eliminated from sewage/ effluent by chemical flocculation.

SECTION 14: TRANSPORTATION INFORMATION

SIN/UN Number:					1593

Shipping Name: 					Paint Related Material Dichloromethane

Packaging Group:					III

Class:						3.3

SECTION 15: REGULATORY INFORMATION

Classification:

R10						Highly flammable

R65						Harmful:  May cause lung damage if swallowed

S2						Keep out of the reach of children (Retail stores only)

S43A						In case of fire, use water fog, foam or powder – Do not use water jets

S62						If swallowed, do not induce vomiting: seek medical advice immediately and

Show this container or label

SECTION 16: ADDITIONAL INFORMATION

Take precautionary measures against static discharges.  In case of any discomfort always seek medical advice. All information is given in good faith but without guarantee in respect of accuracy.  No responsibility is accepted for errors or omissions or the consequences thereof.